# Lithium-Ion Batteries: Chemistry, Materials and Performance

A short technical overview prepared as a sample document for document parsing.

# 1. History

The first rechargeable lithium cell was built in the 1970s at Exxon by M. Stanley Whittingham, who used a titanium disulfide cathode. In 1980 John B. Goodenough showed that lithium cobalt oxide gives a much higher voltage, and in 1985 Akira Yoshino at Asahi Kasei replaced the reactive lithium metal anode with carbon. The three shared the Nobel Prize in Chemistry 2019.

Lithium-ion batteries made portable electronics and modern electric vehicles possible by combining high energy density with long cycle life.

## 1.1 Key milestones

Table 1. Milestones in lithium-ion battery technology

| Year | Event | Organisation | People |
| --- | --- | --- | --- |
| 1976 | Lithium metal / TiS₂ rechargeable cell | Exxon | M. S. Whittingham |
| 1980 | LiCoO₂ cathode proposed | University of Oxford | J. B. Goodenough |
| 1985 | Carbon anode, first practical Li-ion prototype | Asahi Kasei | A. Yoshino |
| 1991 | First commercial Li-ion cell | Sony | — |
| 1997 | LiFePO₄ (olivine) cathode reported | University of Texas at Austin | J. B. Goodenough, A. K. Padhi |
| 2019 | Nobel Prize in Chemistry | Royal Swedish Academy of Sciences | Goodenough, Whittingham, Yoshino |

# 2. Electrochemistry

## 2.1 Electrode reactions

During discharge, lithium ions leave the graphite anode and are inserted into the cathode. For a LiCoO₂ / graphite cell:

Cathode:

Li1−xCoO2 + xLi+ + xe− ⇌ LiCoO2

Anode:

LixC6 ⇌ C6 + xLi+ + xe−

## 2.2 Capacity and energy

The theoretical specific capacity of an electrode material follows from Faraday's law:

Qth = nF3.6M

where:

- n — number of electrons transferred per formula unit;

- F — Faraday constant, 96 485 C/mol;

- M — molar mass of the active material, g/mol;

- the factor 3.6 converts C/g to mAh/g.

The energy delivered by a cell is the integral of voltage over the charge passed:

E = 0QVq dq ≈ Vavg · Q

The charge or discharge current is often expressed as a C-rate: at 1C a cell is fully discharged in one hour, at 2C in 30 minutes, and in general:

I = C · Qnom,    t = 1C h

# 3. Cathode materials

The cathode largely determines cell voltage, capacity, cost and safety. Cobalt-rich layered oxides offer high energy density, while LiFePO₄ trades capacity for thermal stability and cycle life.

Table 2. Common cathode materials (approximate values)

| Structure | Material | Nominal cell voltage, V | Specific capacity, mAh/g |  | Typical applications |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Theoretical | Practical |  |
| Layered oxide | LiCoO₂ (LCO) | 3.7 | 274 | ≈ 150 | Phones, laptops |
|  | NMC811 | 3.7 | ≈ 275 | ≈ 200 | Electric vehicles |
| Spinel | LiMn₂O₄ (LMO) | 3.7 | 148 | ≈ 110 | Power tools |
| Olivine | LiFePO₄ (LFP) | 3.2 | 170 | ≈ 160 | EVs, stationary storage |

Figure 1. Theoretical vs practical specific capacity of cathode materials.

# 4. Computing theoretical capacity

The formula from Section 2.2 in Python:

F = 96485.0  # Faraday constant, C/mol

def theoretical_capacity(molar_mass: float, n: int = 1) -> float:

return n * F / (3.6 * molar_mass)  # mAh/g

print(f"LiFePO4: {theoretical_capacity(157.76):.1f} mAh/g")  # 169.9 mAh/g

# 5. Summary

- Lithium-ion technology was developed between 1976 and 1991 and commercialised by Sony.

- A cell consists of:

  - a graphite anode;

  - a transition-metal oxide or phosphate cathode;

  - a liquid electrolyte with a lithium salt, usually LiPF₆.

- Cathode choice trades off energy density, cost, safety and cycle life.

## Glossary

Specific capacity — charge stored per unit mass of active material, mAh/g.

Intercalation — reversible insertion of ions into a host crystal structure without destroying it.

SEI — solid electrolyte interphase, a passivation layer formed on the anode during the first cycles.

The prize was awarded “for the development of lithium-ion batteries”.

Practical capacities depend on the upper cut-off voltage and on cell design; the table lists typical values.